Multi-objective optimization of I-section steel frames under TCVN 5575:2024: A comparative study of metaheuristic algorithms

Abstract. This study addresses the multi-objective design optimization of welded I-section steel frames in stringent compliance with current TCVN 2737:2023 and TCVN 5575:2024 standards. By integrating a custom Finite Element Method (FEM) solver with non-linear penalty function techniques, three metaheuristic algorithms (NSGA-II, PESA-II, MOPSO) are employed to resolve two conflicting design criteria: the minimization of structural steel weight and lateral frame displacement. Analysis of the resulting Pareto fronts demonstrates clear variances in search space exploration capabilities, where "knee-point" identification facilitates the selection of design configurations that are optimal for practical application. Performance assessments reveal the comprehensive superiority of the MOPSO algorithm, which consistently identifies the lightest structural solutions while exhibiting significantly higher convergence rates. These findings substantiate the robust potential for implementing MOPSO within professional steel structural design consultancy.

Keywords: Multi-objective optimization; steel frames; pareto front; TCVN 5575:2024; metaheuristic; MOPSO; knee-point

# Introduction

Designing steel frames fundamentally requires balancing material economy with lateral stiffness. Because traditional single-objective approaches often yield overly flexible structures, Multi-Objective Optimization (MOO) via metaheuristics is now widely adopted to generate continuous Pareto trade-offs rather than rigid, single-point solutions.In current Vietnamese engineering practice, strict compliance with the newly enacted TCVN 5575:2024 [1] introduces severe non-linear constraints—specifically regarding member slenderness, local plate buckling, and out-of-plane stability. These stringent requirements transform the feasible design space into a highly complex and discontinuous domain. To navigate these practical code boundaries, this research develops an automated computational framework linking an in-house Finite Element Method (FEM) engine with three foundational multi-objective solvers: NSGA-II [3], MOPSO [4], and PESA-II [5]. While newer swarm intelligence variants demonstrate rapid convergence on unconstrained mathematical functions, their actual reliability under heavily codified structural environments demands rigorous verification. Therefore, we deliberately select these three well-documented algorithms to ensure the mathematical accuracy of the derived Pareto fronts. This comparative benchmarking not only identifies the most efficient solver for I-section geometries under the new standard but also establishes a reliable baseline to evaluate future algorithmic advancements in structural optimization [6].

# Framework and Computational Methods

## Mathematical Formulation of the Structural Problem

The bi-objective optimization of frame mass and lateral drift is driven by an eight-dimensional continuous vector defining member cross-sections. To satisfy rigorous strength, global stability, and local buckling constraints mandated by TCVN 5575:2024, a non-linear static penalty function with a large multiplier (R) penalizes constraint violations. An in-house FEM solver evaluates these dynamic responses to guide three established metaheuristics (NSGA-II, MOPSO, PESA-II). For unbiased statistical benchmarking, each algorithm executes 30 independent runs, uniformly utilizing 100 individuals over 200 generations.

|  | X={x1,x2,x3,x4,x5,x6,x7,x8}T | (1) |
| --- | --- | --- |

Where x1,x2,x3,x4  denote the flange width, web depth, web thickness, and flange thickness of the column cross-section, respectively, while x5,x6,x7,x8 represent the corresponding geometric parameters of the beam section.

|  | f1x=ρi=1NeAix⋅Li | (2) |
| --- | --- | --- |

Where ρ denotes the mass density of the steel material; Aix and Li represent the cross-sectional area and length of the i -th element, respectively; and Ne indicates the total number of elements constituting the frame system.

|  | f2x=Δmaxx | (3) |
| --- | --- | --- |

Where Δmaxx defines the maximum lateral displacement value at the top node of the frame, extracted directly from the finite element method (FEM) solver.

|  | Fkx=fkx+Rj=112max0,gjx2 | (4) |
| --- | --- | --- |

Where Fkx and fkx denote the penalized and original objective functions for the k-th criterion, respectively; R represents the strictly large static penalty parameter; and gjx signifies the j-th behavioral constraint evaluated within the framework.

## Integration and configuration of metaheuristic algorithms

The core of the numerical simulation relies on an in-house Finite Element Method (FEM) code linked with a MATLAB-based TCVN 5575:2024 compliance checking module, utilizing the Yarpiz framework. Throughout the iterative search process, mathematical design vectors are translated into physical structural parameters. By computing member internal forces and global displacements, the solver evaluates constraint violations via a penalty mechanism, which subsequently directs the search direction of the algorithms. For a fair comparative analysis, NSGA-II, MOPSO, and PESA-II were assigned identical execution constraints: a population of 100 agents spanning 200 generations. Furthermore, to mitigate the random variations typical of evolutionary computing, the optimization routines were repeated for 30 independent trials, ensuring the statistical validity of the final Pareto sets.

Table 1. Specific computational variables and initial configurations driving the NSGA-II, MOPSO, and PESA-II routines.

| Item | Algorithm parameters | MOPSO | NSGA-II | PESA-II |
| --- | --- | --- | --- | --- |
| 1 | Population size (Npop​) | 100 | 100 | 100 |
| 2 | Maximum iterations (Itermax​) | 200 | 200 | 200 |
| 3 | Inertia weight (w) | 0.9 | - | - |
| 4 | Acceleration coefficients (c1​,c2​) | 2.0, 2.0 | - | - |
| 5 | Crossover rate (pcross​) | - | 0.7 | 0.5 |
| 6 | Mutation rate (pmut​) | 0.5 | 0.3 | 0.5 |
| 7 | External grid size | 7x7 | - | 7x7 |

# Experimental results and discussion

## Model description and FEM validation

To evaluate the performance of the integrated metaheuristic frameworks, a benchmark two-story, single-span steel plane frame is analyzed under the joint actions of vertical gravity loads and regional equivalent wind pressures. Prior to embedding this structural model into the evolutionary optimization loops, the accuracy of the in-house MATLAB-based Finite Element Method (FEM) code required systematic verification against the industry-standard commercial software SAP2000. The cross-validation process yielded a maximum relative error in nodal displacement of only 0.0445%. This minimal numerical discrepancy confirms that the developed analytical core possesses the necessary precision and reliability for evaluating structural response and objective metrics throughout the iterative optimization process.

Table 2. Summary of design variables and input constraints for the numerical frame simulation.

| No. | Model parameters | Symbol | Applied value |
| --- | --- | --- | --- |
| 1 | Main span / Frame spacing | L / B | 10.0 m / 6.0 m |
| 2 | Story 1 height / Story 2 height | H1​ / H2​ | 4.0 m / 4.0 m |
| 3 | Total vertical load (dead + live loads) | qv​ | 10.0 kN/m² |
| 4 | Basic wind pressure [2] | W0​ | 1.55 kN/m² |
| 5 | Yield strength (SS400 steel) | fy​ | 245 MPa |
| 6 | Elastic modulus (SS400 steel) | E | 210000 MPa |

Fig. 1. Frame configuration and loading diagram [2]

## Pareto front exploration and knee-point identification

The non-dominated solutions obtained by all three algorithms successfully outline a well-defined convex Pareto front, reflecting the trade-off behavior between frame mass and lateral stiffness. However, their local exploration patterns reveal clear operational distinctions. MOPSO shows absolute dominance in exploring the extreme low-mass boundaries of the search space, capturing the absolute lightest structural design at 2064.84 kg (with a corresponding lateral drift of 10.91 mm). This solution outperforms the minimum weight configurations found by NSGA-II (2077.31 kg) and PESA-II (2128.14 kg). Conversely, while PESA-II pushes the front toward the stiffest extreme-achieving a minimum horizontal displacement of 3.37 mm-the associated mass penalty increases dramatically to 6205.99 kg, making such a configuration economically impractical. From a practical engineering design perspective, the numerical data exhibits high mathematical consistency. Both MOPSO and NSGA-II independently converge toward an identical "knee-point" region on the Pareto front. Positioned at a structural mass of approximately 3090 kg and a lateral displacement of 6.7 mm, this specific knee-point solution offers a highly balanced compromise, optimizing structural safety and cost-efficiency for industrial engineering applications.

Fig. 2. Distribution of non-dominated solutions and structural knee-points within the trade-off space of frame mass versus lateral drift

## Computational Efficiency Benchmarking

Coupling iterative finite element evaluations with metaheuristic loops typically imposes a severe computational burden. Statistical assessments across 30 independent trials highlight the processing efficiency of the MOPSO framework. Yielding an average execution time (tmean) of 16.71 seconds, MOPSO accelerates the automated workflow by a factor of 3.45 against the NSGA-II baseline (57.64 s) and 2.94 against PESA-II (49.19 s). Although the stochastic velocity updates inherent to particle swarms induce a slightly elevated standard deviation (σ= 2.16 s), this aggressive convergence characteristic confirms MOPSO as a highly practical solver for complex, code-constrained structural optimization tasks.

Table 3. Statistical metrics of computational runtimes (in seconds) over 30 independent runs.

| Algorithm | Mean time (tmean​) | Standard deviation (σ) | Fastest run (tmin​) | Slowest run (tmax​) |
| --- | --- | --- | --- | --- |
| MOPSO | 16.71 | 2.16 | 11.18 | 20.10 |
| NSGA-II | 57.64 | 1.64 | 56.59 | 65.93 |
| PESA-II | 49.19 | 1.54 | 46.30 | 52.67 |

# Conclusions

This study establishes an automated, FEM-based optimization framework for I-section steel frames strictly governed by TCVN 5575:2024. Computational benchmarking yields the following conclusions:

Solver Efficiency: MOPSO demonstrated superior search capabilities and speed. It identified the absolute minimum-weight configuration (2064.84 kg) and converged 3.45 times faster than the NSGA-II baseline, proving highly effective for constrained, non-linear structural design tasks.

Practical Design Application: All evaluated algorithms successfully mapped the convex Pareto boundary, explicitly reflecting the mass-stiffness trade-off. The consensus "knee-point"-identified at 3090 kg with a 6.7 mm lateral drift-provides a mathematically rigorous compromise solution for engineering practice.

This baseline methodology is readily scalable to complex spatial structures. Subsequent research will integrate geometric stiffness matrices for non-linear P-Δ effects and formulate Reliability-Based Design Optimization (RBDO) protocols to address material and load uncertainties.

# References

- Ministry of Construction, TCVN 5575:2024: Steel structures - Design standard. Construction Publishing House; 2024.

- Ministry of Construction, TCVN 2737:2023: Load and forces - Design standard. Construction Publishing House; 2023.

- Deb, K., Pratap, A., Agarwal, S., Meyarivan, T.: A fast and elitist multiobjective genetic algorithm: NSGA-II. IEEE Transactions on Evolutionary Computation 6(2), 182-197 (2002).

- Coello Coello, C.A., Pulido, G.T., Lechuga, M.S.: Handling multiple objectives with particle swarm optimization. IEEE Transactions on Evolutionary Computation 8(3), 256-279 (2004).

- Corne, D.W., Jerram, N.R., Knowles, J.D., Oates, M.J.: PESA-II: Region-based selection in evolutionary multiobjective optimization. Proceedings of the Genetic and Evolutionary Computation Conference (GECCO-2001), 283-290 (2001).

- Kaveh, A.: Advances in Metaheuristic Algorithms for Optimal Design of Structures. Springer Cham; 2021.